# Fermi National Accelerator Laboratory

## General Information for Manufacture of Printed Circuit Boards

Note: All dimensions are in inches unless specified

| ### “MPPC Amp” board, ver. October 21, 2016 Pcb manufacturing and assembling | Board Name |
| --- | --- |
| 600/700/800 | Quantity |
| 4 Layer (FR-4/GFN) | Board Type (# signal layers - # power layers) |
| 0.063” | Board Thickness and Tolerance |
| 1.024” x 1.024” | Board Dimensions and shape |
| SMOBC / ENIG | Finish/Plating (smobc) |
| 1 oz. | Cu Weight outer layers (ounces per square foot) |
| 0.5 oz. | Cu Weight inner layers (ounces per square foot) |
| 0 | Number of gold contacts/side |
| 0.008” | Minimum Trace width & tolerance |
| 0.012” | Minimum Trace clearance & tolerance |
| Top & bottom , color Green | Solder Mask ( top & bottom – top only) |
| LPI | Solder Mask Type (wet/dry/LPI) |
| Top & bottom - White | Silk Screen ( top & bottom – top only - white) |
| Yes / net list from Gerbers | Testing Required (yes/no/specify) |
| 57/ 4 | Number of Surface mount pads TOP and BOT (only needed for testing) |
| 36 | Number of Through Holes |
| 4 | Number of Drill Sizes |
| 0.010” | Smallest Hole |
| 0.091” | Largest Hole |
| 27 | Number of epoxy filled and overplated vias |
| 0 | Number of Blind Vias |
| 0 | Number of Buried Vias |
| No | Controlled Impedance (yes/no/specify) |
| --------------------------------- | Physical Board Stackup |
| .TSK | Top silk layer |
| .TMK | Top mask layer |
| .TOP | Top layer (Signal) 1oz copper / 0.010” FR-4 |
| .LR2 | 2nd layer (Signal) 0.5oz copper / 0.040” FR-4 |
| .LR3 | 3rd layer (Signal) 0.5oz copper / 0.010” FR-4 |
| .BOT | Bottom layer (Signal) 1oz copper |
| .BMK | Bottom mask |
| .BSK | Bottom silk |
| .DRL | Drill layer |
| .NCD | NCD drill file / plated |
| .BRD | Board outline |
| .TAS | Top assembly |
| TAS.PDF | Top assembly / PDF |
| BOM.XLSX | Bill of materials |
| .PNP | X/Y, pick and place ASCII file |
| Gerber files via email | Material Supplied |
| 10 days turn for printed circuit boards, 10 days turn for assembly (with one pilot panel assembled and delivered for testing before the main assembly starts) | Required Delivery |
| Solder flux should be removed from the boards regardless of its type | Special NOTES |
| Sergey Los (630)840-8313, los@fnal.gov | Technical Contact Person and ph. number |